# Executive Briefing: iNAND/eMMC Subsystem Design & UVM Verification Plan

# 1. Executive Summary & Project Scope

This document outlines the architectural design and the rigorous Universal Verification Methodology (UVM) verification plan for our eMMC/iNAND memory controller interface. The primary objective is to transition from legacy test benches to an industry-standard, robust UVM framework capable of validating block-level read and write transactions (CMD17 and CMD24) under strict timing constraints, achieving zero simulation mismatches (UVM_ERROR: 0).

# 2. eMMC Controller Architectural Overview

The Design Under Test (DUT) interfaces with the memory subsystem using a synchronous command-and-data protocol optimized for 512-byte block granularity.

## Core Interface Signals & Protocol Operation

cmd_stb & cmd_code: Command strobe and opcode qualifier. Initiates transaction execution (e.g., CMD17 for single block read, CMD24 for single block write).

lba (Logical Block Address): 32-bit address bus designating the target block location in memory.

Write Data Path (wr_en, dat_in): Gated by write enable handshaking, transferring a deterministic 512-byte payload sequentially.

Read Data Path (dat_vld, dat_out, ready): Flow-controlled output bus streaming back the 512-byte payload with cycle-accurate validity gating.

# 3. UVM Verification Environment Architecture & Flow

To ensure high observability, controllability, and cycle-accurate verification, the UVM architecture implements a reactive monitoring loop and automated reference modeling:

UVM Verification Flow Diagram:
+-------------------+      +-------------------+      +-------------------+
|   UVM Sequence    | ---> |    UVM Driver     | ---> |   Virtual IF      | 
| (Stimulus Gen)    |      |  (drv_cb, 1ns)    |      | (Clocking Blocks) |
+-------------------+      +-------------------+      +-------------------+
                                                                |
                                                                v
+-------------------+      +-------------------+      +-------------------+
|    Scoreboard     | <--- |   UVM Monitor     | <--- |     DUT           | 
| (Reference Model) |      | (Lock-and-Burst)  |      | (eMMC Controller) |
+-------------------+      +-------------------+      +-------------------+

| Component | Architecture & Responsibility |
| --- | --- |
| Sequence Item (inand_sequence_item) | Encapsulates transaction attributes (cmd, lba, and dynamic payload byte data[]) utilizing UVM Factory registration. |
| Driver (inand_driver) | Translates high-level sequence transactions into cycle-accurate bus operations via drv_cb clocking blocks with controlled 1ns skews. |
| Monitor (inand_monitor) | Employs a "Lock-and-Burst" protocol-aware algorithm: uses a dynamic while loop to catch initial handshakes without skew, followed by a deterministic 512-cycle for burst loop. |
| Scoreboard (inand_scb) | Maintains a reference memory model (ref_memory) indexed by LBA. Cross-checks incoming read transactions against previously written data. |

# 4. Comprehensive Verification Plan: End-to-End eMMC Use Case

Rather than treating stimulus as isolated unit tests, the verification strategy evaluates the controller through a comprehensive End-to-End Traffic and Data Integrity Use Case. This scenario exercises the full pipeline through progressive operational phases:

| Use Case Phase | Operational Objective | Description & Detailed Verification Behavior |
| --- | --- | --- |
| Phase 1: Basic Sanity Read/Write (TST_BASIC_RW) | Functional Sanity Check | Executes sequential single-block write (CMD24) followed immediately by a read (CMD17) on identical and adjacent LBAs to establish baseline protocol compliance. |
| Phase 2: Address Randomization (TST_RAND_LBA) | Address Space Stress | Stress-testing random LBA distribution across the addressable range, verifying the reference model’s ability to track sparse and non-contiguous memory allocations. |
| Phase 3: Pipeline & Flow Control (TST_BACK_TO_BACK) | Bandwidth & Readiness | Executes consecutive read/write requests without idle cycles on the bus, testing the controller’s ready handshake signals and internal queue buffering under high throughput. |
| Phase 4: Payload Integrity (TST_BOUNDARY_ALIGN) | Byte-Level Boundary Check | Verifying exact byte alignment and edge-case preservation from Byte[0] through Byte[511] (specifically stabilizing Byte[511] handshakes) to eliminate shift errors. |

## Functional Coverage Goals

• Command Coverage: 100% coverage on supported opcodes (CMD17, CMD24).
• Address Coverage: Verification of minimum, maximum, and intermediate LBA boundaries.
• Data Integrity Coverage: 100% payload bit/byte match across all test transactions.

# 5. Current Status, Metrics, and Next Steps

• Current Simulation Health: Regression tests executing 16–20 transactions currently achieve clean execution passing metrics with zero structural simulation errors (UVM_ERROR: 0).
• Resolved Technical Bottlenecks: Addressed phase skew and off-by-one index shifts in monitor data sampling; stabilized block-end edge conditions for Byte[511] via reactive handshake synchronization.
• Next Phase Milestones: Expand stimulus generation into constrained-random sequence generators; introduce multi-block burst transfers (CMD18 / CMD25) to scale protocol bandwidth verification.